# ANNEX 3.2-B: TRAFFIC PRIORITISATION & QoS MANAGEMENT

Purpose: To define traffic priority classes and failover protocols ensuring critical communications are maintained during network congestion or failure.

## Section 1: Traffic Priority Classification

| Priority | Traffic Class | Description | Bandwidth % | Examples |
| --- | --- | --- | --- | --- |
| P1 - Critical | Command & Control | Emergency coordination | 40% | Disaster alerts, SAR comms |
| P2 - High | Voice | Real-time voice | 25% | VoIP, radio relay |
| P3 - Medium | Operational Data | Critical data transfer | 20% | SitReps, GIS data |
| P4 - Standard | General Data | Normal operations | 10% | Email, file sharing |
| P5 - Low | Background | Non-essential | 5% | Updates, backups |

## Section 2: QoS Parameters

| Parameter | P1-P2 Target | P3-P4 Target | Measurement Method |
| --- | --- | --- | --- |
| Latency | < 150 ms | < 300 ms | Ping test |
| Jitter | < 30 ms | < 50 ms | Network analyser |
| Packet Loss | < 0.5% | < 2% | Throughput test |
| Availability | > 99.9% | > 99% | Uptime monitoring |

## Section 3: Failover Priority Matrix

| Failure Scenario | Primary Action | Secondary Action | Recovery Time Target |
| --- | --- | --- | --- |
| Primary VSAT failure | Switch to Starlink | Activate BGAN backup | < 30 seconds automatic |
| Total SATCOM failure | Activate LTE backup | Deploy HF/VHF relay | < 5 minutes manual |
| Power failure | UPS automatic switch | Generator startup | < 10 seconds UPS |
| Bandwidth congestion | Traffic shaping active | P4-P5 traffic throttled | Immediate |
| Cyber incident | Isolate affected segment | Switch to backup VPN | < 15 minutes |

## Section 4: QoS Configuration Verification

| Configuration Item | Status | Verified By |
| --- | --- | --- |
| Traffic classification rules configured on router | ☐ Complete |  |
| Bandwidth allocation percentages applied | ☐ Complete |  |
| Failover triggers tested and documented | ☐ Complete |  |
| Monitoring alerts configured | ☐ Complete |  |