# Annex 6.2-C: Regional VPN Configuration Standards

## VPN Configuration Standards

| Parameter | Standard Configuration |
| --- | --- |
| Protocol | IPsec/IKEv2 (Site-to-Site); SSL/TLS (Remote Access) |
| Encryption Algorithm | AES-256-GCM |
| Hash Algorithm | SHA-384 or SHA-512 |
| Key Exchange | Diffie-Hellman Group 20 (384-bit ECDH) or higher |
| Authentication | Certificate-based with RSA-4096 or ECDSA-P384 |
| Perfect Forward Secrecy | Enabled (mandatory) |
| IKE Lifetime | 8 hours (28,800 seconds) |
| IPsec SA Lifetime | 1 hour (3,600 seconds) |
| Dead Peer Detection | Enabled; 30-second interval |
| NAT Traversal | Enabled (UDP 4500) |
| Split Tunneling | Disabled for SHOC networks |
| MFA Requirement | Mandatory for all remote access VPN |

## Partner VPN Connections

| Partner | VPN Type | Remote Gateway | Status | Last Verified |
| --- | --- | --- | --- | --- |
| SADC Secretariat | Site-to-Site |  |  |  |
| SADC Climate Services Centre | Site-to-Site |  |  |  |
| SADC Regional EW Centre | Site-to-Site |  |  |  |
| Botswana OSOCC | Site-to-Site |  |  |  |
| AU Continental Situation Room | Site-to-Site |  |  |  |
| AMHEWAS Regional Node | Site-to-Site |  |  |  |
| IFRC Regional Office | Site-to-Site |  |  |  |
| WFP Regional Bureau | Site-to-Site |  |  |  |
| UNDP Regional Office | Site-to-Site |  |  |  |

## SHOC VPN Network Segments

| Network Segment | Subnet | Purpose | Access Level |
| --- | --- | --- | --- |
| SHOC-CORE | 10.10.1.0/24 | Core infrastructure servers | Restricted |
| SHOC-EW | 10.10.2.0/24 | Early warning systems | Restricted |
| SHOC-OPS | 10.10.3.0/24 | Operations coordination | Restricted |
| SHOC-ADMIN | 10.10.4.0/24 | Finance, HR, Admin | Confidential |
| SHOC-PARTNER | 10.10.5.0/24 | Partner access zone | Controlled |
| SHOC-DMZ | 10.10.6.0/24 | Public-facing services | Public |
| SHOC-MGMT | 10.10.254.0/24 | Network management | Secret |