# Annex 8.4-L: Satellite Tracking Device Specifications

Purpose: Define minimum technical specifications for satellite tracking devices fitted to SHOC fleet vehicles and high-value cargo shipments, ensuring real-time visibility per UNDAC Handbook logistics protocols and SHOC ICT Policy.

Reference: SOP Section 2.5 │ SHOC ICT Policy v3.0 │ Output: SAT-SPEC-01

## Minimum Technical Requirements

| Parameter | Minimum Specification | Preferred Specification |
| --- | --- | --- |
| Tracking Technology | GPS + GLONASS | GPS + GLONASS + Galileo |
| Position Accuracy | ≤5 metres CEP | ≤2.5 metres CEP |
| Update Frequency | Every 60 seconds (moving) | Every 30 seconds (moving) |
| Communication | GSM/GPRS with satellite fallback | Dual-mode cellular + Iridium satellite |
| Battery Backup | ≥8 hours internal battery | ≥24 hours internal battery |
| Operating Temperature | -10°C to +60°C | -20°C to +70°C |
| Ingress Protection | IP65 minimum | IP67 or higher |
| Geofencing | Configurable zones with alerts | Unlimited zones with corridor alerts |
| Data Interface | API integration with DFMS | API + direct ERP/DLMS feed |
| Tamper Detection | Alert on disconnection | Alert + photo capture |

## Deployment Standards

All SHOC-owned vehicles and hired vehicles on missions exceeding 48 hours must be fitted with compliant devices. High-value cargo (CAT-1, CAT-2) shipments require dedicated cargo trackers. Device data feeds the Fleet Dashboard (8.4-N) and DLMS.

| Role | Signature | Date |
| --- | --- | --- |
| Prepared By: | _______________ | _______________ |
| Reviewed By: | _______________ | _______________ |
| Approved By: | _______________ | _______________ |